# EX. NO: 8 — OS NMAP LIVE HOST DISCOVERY

Date: 24/9/26

# Aim:

Discover live/active hosts in a network using scans such as ARP, ICMP, TCP and UDP.

# Objective:

To understand how Nmap identifies active systems before performing further network scanning operations.

# Software:

## Nmap

Try hack Nmap from a computer with network access.

# Theory:

Nmap is a free and open-source network scanning tool used to discover live hosts and identify network services. It was created by Gordon Lyon.

Nmap uses different live-host discovery methods using different protocols.

## ARP Scan

Uses ARP scan requests to find hosts on the same subnet.

## ICMP Scan

Uses ICMP requests and replies to identify hosts.

## TCP Scan

Uses TCP requests and packets to specified ports and checks the response.

## UDP Scan

Sends UDP packets and can identify an active host through an ICMP port unreachable response.

# Observation:

  - ARP request can discover hosts within the same subnet.

  - ARP cannot cross routers because it operates at the link layer.

  - An ACK ping generally requires a predefined port.

  - UDP ping can identify an active host when an ICMP port unreachable is received.

  - The -sn option allows Nmap to perform host discovery without conducting a port scan.

# Result:

Thus, the live host discovery using Nmap was successfully studied and performed. The experiment helped in understanding how active hosts can be identified before performing further network scans.